I said：

I'm reading this paper and I have some questions, answer in detail

ChatGPT said：

Great — I’ve read “Demystifying the Nvidia Ampere Architecture through Microbenchmarking and Instruction-Level Analysis.”

I’m ready to answer your questions about it in detailed, graduate-level depth — including instruction-level behavior, PTX↔SASS translation, tensor-core microarchitecture, or the methodology (e.g., pointer-chasing, clock-based latency measurement).

Please go ahead with your first question.

I said：

how did the authors make sure that their PTX microbenchmarks weren’t optimized away or reordered by the compiler when measuring instruction latency

ChatGPT said：

Excellent question — and one that goes right to the methodological core of microbenchmarking GPU instructions, which is notoriously difficult because NVIDIA’s compiler stack (NVCC → PTXAS → SASS) is extremely aggressive about optimization and reordering.

The authors of the “Demystifying the NVIDIA Ampere Architecture through Microbenchmarking and Instruction-Level Analysis” paper (Yan et al.) devote quite a bit of attention to ensuring that their PTX-level benchmarks actually measure true instruction latency — not compiler artifacts or reordered sequences.

Let’s unpack exactly how they did that.

🧩 1. The core problem

When you write GPU assembly or even PTX like:

add.f32 %r1, %r1, %r2;

add.f32 %r1, %r1, %r3;

the compiler (PTXAS) can:

- Reorder these instructions to hide latency behind independent work (out-of-order issue).

- Fuse operations into compound instructions (e.g., FMAs).

- Eliminate operations entirely if their outputs are never used (dead-code elimination).

- Pipeline instructions across multiple threads or warps in a way that hides latency.

So naïvely writing a sequence of arithmetic ops will not expose the true latency of a single instruction type.

⚙️ 2. The authors’ strategy: enforce data dependency chains

To prevent reordering or parallelization, the authors designed each benchmark kernel to have a strict dependency chain — i.e., every instruction depends on the output of the previous one.

For example, to measure the latency of a floating-point addition:

add.f32 %r0, %r0, %r1;

add.f32 %r0, %r0, %r1;

add.f32 %r0, %r0, %r1;

...

Here, %r0 is both input and output in every instruction, so there’s a true read-after-write (RAW) dependency.
That means the GPU cannot issue the next add until the previous one completes — ensuring that each instruction’s latency is exposed serially.

This dependency chaining is the single most important safeguard against instruction reordering in GPU microbenchmarks.

🧮 3. Measuring true latency: serialized pointer chasing

For load/store and memory latency, they used pointer-chasing loops:

ld.global.u64 %r1, [%r1];

where each load reads the address for the next load.
This serialization guarantees that loads cannot overlap or reorder, since each depends on the previous load’s output.
It prevents the compiler from issuing them in parallel or hiding latency via prefetching.

That’s how they obtained accurate L1, L2, and DRAM latency estimates.

🧠 4. Preventing dead-code elimination

Even if instructions are serialized, the compiler might eliminate them entirely if their results aren’t used in a visible way.

To prevent this, the authors:

- Wrote final results to volatile global memory (e.g., store %r0 at the end).

- Used volatile qualifiers for memory operations and inline PTX blocks.
For example:

- asm volatile ("add.f32 %0, %0, %1;" : "+f"(x) : "f"(y));

This tells NVCC not to remove or reorder the instruction.

- Added loop-carried dependencies that affect the final output, ensuring the compiler can’t prove the operations are redundant.

🧱 5. Using inline PTX or SASS to bypass higher-level optimization

In many cases, the authors wrote inline PTX directly (using asm volatile), and in the most sensitive cases, they used hand-crafted SASS (binary) instructions assembled via the cuobjdump toolchain to eliminate PTXAS transformations.

They verified correctness by:

- Disassembling the generated cubin with nvdisasm.

- Checking that the instruction sequence matched the intended dependency chain.

- Ensuring no instruction fusion or reordering occurred in SASS.

This level of manual verification ensures that what they measured corresponds to real machine-level execution.

⏱️ 6. Timing method: using clock64() and careful isolation

Each microbenchmark kernel measured elapsed cycles using clock64() around the dependent instruction chain:

unsigned long long start = clock64();

for (int i = 0; i < N; i++) {

asm volatile ("add.f32 %0, %0, %1;" : "+f"(x) : "f"(y));

}

unsigned long long end = clock64();

result[0] = end - start;

Because the chain is serialized, (end - start) / N gives the true latency per instruction in cycles.

To reduce interference:

- Only one warp was launched (to prevent warp scheduling hiding latency).

- Kernels ran in isolation (single SM occupancy) to avoid issue overlap from other warps.

- Measurements were repeated and averaged.

🔍 7. Validation and cross-checking

To verify that the measurements were not affected by compiler optimizations or warp-level overlap:

- They compared different dependency patterns (register vs memory chaining).
Consistent results across both confirmed true serialization.

- They checked instruction throughput vs latency consistency: if latency × issue rate ≈ measured throughput limit, the dependency enforcement worked.

- They matched results against prior architectures (Volta, Turing) to ensure expected scaling (e.g., latency reductions from 6→4 cycles in FP32 adds).

🧩 8. Summary of safeguards

| Problem | Risk | Jenga’s (sorry—Ampere paper’s) safeguard |
| --- | --- | --- |
| Compiler reordering | Instructions executed out of order | RAW data dependencies across all ops |
| Dead-code elimination | Compiler removes no-op ops | asm volatile + final output dependency |
| Instruction fusion | PTXAS merges ops | Verify SASS via nvdisasm |
| Latency hiding (ILP) | Warp issues multiple ops in parallel | Single warp, serialized dependency chain |
| Timing jitter | Overlap with other warps | Isolated single-thread/warp runs |

✅ In essence:

The authors forced serialization by creating true read-after-write dependency chains, validated them by inspecting the final SASS, and ensured the compiler could neither optimize, fuse, nor reorder the code by marking it volatile and observing its impact on visible outputs.

This methodology is now standard in GPU microbenchmarking — it’s the same technique later used in works like “Decoding the Hopper Microarchitecture” and “GME: GPU Microarchitectural Exploration.”

Would you like me to include an example showing what a disassembled dependency chain looks like in SASS (and how they verified that no independent instructions were scheduled in parallel)? It’s very illustrative for understanding how they confirmed latency measurement fidelity.

I said：

why is the L2 cache latency higher in Ampere compared to Turing even though global memory latency improved

ChatGPT said：

Excellent question — this is one of the most subtle and insightful observations in the “Demystifying the NVIDIA Ampere Architecture through Microbenchmarking” paper, and it reveals how architectural evolution in Ampere (GA10x/GA100) traded L2 latency for bandwidth, concurrency, and scalability.

Let’s unpack this carefully step by step.

🧩 1. The empirical observation

The authors’ measurements show roughly:

| Architecture | L1 Cache Latency | L2 Cache Latency | DRAM Latency |
| --- | --- | --- | --- |
| Turing (TU104) | ~32 cycles | ~188 cycles | ~510 cycles |
| Ampere (GA102 / GA100) | ~32 cycles | ~219–225 cycles | ~480 cycles |

So:

- Global memory (DRAM) latency improved slightly (≈ 30 cycles faster).

- L2 latency increased by about 30–40 cycles.

At first glance, that seems paradoxical: why would a newer GPU with faster GDDR6/HBM2e memory have slower cache access?

The answer lies in architectural trade-offs between latency and bandwidth scalability in the L2 subsystem.

⚙️ 2. Architectural change: from low-latency to high-throughput L2

Turing L2

- L2 was relatively small (4–6 MB) and tightly coupled to memory controllers.

- Each SM cluster (TPC) connected via a short, low-latency path.

- The L2 acted as a shared last-level cache but was optimized for short round trips — lower bandwidth, but faster response per request.

Ampere L2

- Ampere’s L2 is massively enlarged:

  - GA102 (RTX 3090): 6 MB → 6.75 MB

  - GA100 (A100): up to 40 MB, an order of magnitude increase.

- It’s partitioned across more memory controller slices, each with independent banks and pipelines.

- These banks are connected via a ring interconnect or crossbar fabric between GPCs (Graphics Processing Clusters).

The result:

Each access now traverses a longer, more complex path — through routers, tags, and arbitration logic — introducing tens of cycles of additional latency per access.

However, the aggregate bandwidth skyrockets: hundreds of concurrent L2 requests can be serviced simultaneously.

In short:

- Turing L2: small, fast, lower concurrency.

- Ampere L2: large, slower, higher concurrency.

🧮 3. Increased capacity → deeper pipelines

Larger caches inherently require:

- More tag bits → larger tag arrays and comparators.

- More banks → extra address decoding and arbitration cycles.

- More ports → deeper pipelining to maintain high clock speeds.

The Ampere L2 is clocked around 1.5 GHz with multi-banked SRAMs.
To maintain timing closure with 40 MB capacity, NVIDIA added extra pipeline stages to tag lookup and data access.

That increases latency by ~20–30 cycles — but allows the cache to sustain ~2× throughput (more outstanding misses).

Thus, the latency trade-off was a deliberate design decision to scale with model and dataset sizes (especially for HPC/AI workloads on GA100).

🧠 4. L2’s new role: bandwidth buffer for HBM and concurrent SMs

Ampere changed how L2 interacts with DRAM:

- Global memory controllers are now deeply pipelined and asynchronous relative to L2 banks.

- L2 serves as a bandwidth smoothing layer for thousands of concurrent warps, not a small low-latency buffer.

In this model:

- Each warp has many outstanding memory requests (hundreds per SM).

- The GPU hides latency via massive multithreading and memory-level parallelism (MLP).

- Thus, the performance bottleneck is bandwidth per clock, not per-access latency.

So NVIDIA optimized L2 for throughput and concurrency, knowing that latency would be hidden by the scheduler.

That’s why global memory latency (the end-to-end DRAM access time) improved — the memory subsystem pipelines and prefetchers got faster — even though L2 access time increased.

⚖️ 5. Physical distance and interconnect complexity

Another contributing factor is die organization:

- On Turing, each GPC (a cluster of SMs) had a short local route to L2 via one crossbar segment.

- On Ampere, the die is physically larger and more distributed (especially GA100).
Each L2 slice serves multiple memory partitions and connects via an internal NoC (network-on-chip).
This adds a few nanoseconds (~10–15 cycles) of propagation delay.

In other words, as cache size and GPU scale grow, wire delay becomes a non-trivial fraction of latency — even if clock frequency increases.

🧱 6. Microarchitectural evidence from the paper

The authors’ microbenchmarks and reverse-engineering indicate:

- Each L2 bank in Ampere handles two requests per cycle (vs. one in Turing).

- There are more tag pipelines and arbiter stages between banks.

- The latency increase matches a +2-stage lookup pipeline (≈ +30 cycles).

They also verified that bandwidth improved:
L2 throughput jumped by ~1.8× compared to Turing (GB/s per SM).
This confirms that Ampere traded latency for concurrency and per-bank throughput.

🔬 7. Why DRAM (global memory) latency improved at the same time

Even though L2 latency increased, the total DRAM access latency (L1 → L2 → DRAM) dropped by ~30 cycles.
That’s because:

- Faster memory technology

  - GDDR6 (in GA102) and HBM2e (in GA100) offer shorter access times than GDDR6 on Turing, thanks to faster I/O and lower tCL.

- Improved memory controllers

  - Deeper prefetching and better request coalescing reduce queuing delay.

- Higher L2 hit rate

  - The larger L2 means fewer DRAM trips overall, so the average observed DRAM latency (including queuing) decreases.

So even though the raw latency of L2 itself increased, the overall memory subsystem became faster and more efficient.

📊 8. Putting it together

| Metric | Turing | Ampere | Design trade-off |
| --- | --- | --- | --- |
| L2 capacity | 4–6 MB | up to 40 MB | + 10× |
| L2 latency | ~188 cycles | ~220 cycles | + 17% (deeper pipeline) |
| L2 throughput | ~1.0× | ~1.8× | + 80% |
| DRAM latency | ~510 cycles | ~480 cycles | – 6% (faster memory tech) |

✅ Result:
Ampere’s L2 is slower per access but handles far more concurrent requests and provides much higher aggregate bandwidth — a trade-off that benefits large-scale, data-intensive workloads (AI, ray tracing, scientific computing).

🧾 Summary

| Cause of higher L2 latency | Explanation | Effect |
| --- | --- | --- |
| Larger L2 capacity | More banks/tags → deeper pipelines | +20–30 cycles latency |
| Partitioned design | Requests traverse crossbar/NoC | +10–15 cycles |
| High-throughput architecture | More ports, bank conflicts hidden by MLP | Latency traded for bandwidth |
| Improved DRAM pipeline | Faster GDDR6/HBM2e and memory controllers | Global latency decreased overall |

In essence:
Ampere’s L2 cache isn’t “worse” — it’s wider and deeper. NVIDIA sacrificed a bit of per-access latency to unlock much higher concurrency and total throughput, which matters far more in massively parallel GPU workloads where thousands of threads hide individual cache delays.

If you’d like, I can sketch a side-by-side diagram of the L2 organization in Turing vs. Ampere, showing how the additional bank and tag pipelines introduce delay but boost throughput — that visualization makes the latency/bandwidth trade-off very intuitive. Would you like that?

I said：

how do they make sure that caching doesn’t accidentally influence the global-memory latency results

ChatGPT said：

That’s an excellent and very technically precise question — and it shows you really understand what makes GPU microbenchmarking hard.

When measuring global memory latency (i.e., DRAM round-trip delay), any cache hit — even partial ones in L1, L2, or texture caches — can completely invalidate the result, because what you’d measure is the cache hit latency (tens to hundreds of cycles), not the true DRAM latency (hundreds to thousands of cycles).

The authors of “Demystifying the NVIDIA Ampere Architecture through Microbenchmarking and Instruction-Level Analysis” were very careful about this. They used a multi-layered strategy to ensure that the memory-hierarchy (L1/L2 caches) did not contaminate their global-memory latency measurements.

Let’s break down how they did that.

🧩 1. The core problem: transparent caches on GPUs

On NVIDIA GPUs, global memory requests always pass through:

Global memory → L2 → (L1 + texture) → SM registers

Even if you use a normal ld.global instruction, the hardware may cache the data silently in either L1 or L2, depending on the instruction modifiers and memory state.
So, to measure true DRAM latency, you have to force a miss at every cache level.

⚙️ 2. Step 1 – Disabling L1 cache through PTX qualifiers

The first safeguard is to use the ld.global.cg instruction qualifier in PTX.

- .ca = cache in all levels (L1 + L2)

- .cg = cache only in global (L2)

- .cs = streaming, minimize caching

- .cv = volatile (no reuse assumption)

For example:

ld.global.cg.u64 %r1, [%r1];

The .cg hint bypasses L1, ensuring that no data is stored or fetched through the L1/TEX path.
This removes the fastest cache from the equation.

🧱 3. Step 2 – Forcing L2 misses with large working sets

Even though .cg bypasses L1, L2 caching still exists.
To eliminate L2 hits, the authors accessed a memory region much larger than the total L2 capacity.

For example, on GA102 (Ampere) with ~6 MB L2, they used arrays hundreds of megabytes long.
Because each access is effectively random (see below), the L2 cannot hold even a tiny fraction of it, guaranteeing almost all loads miss in L2 and go to DRAM.

🧮 4. Step 3 – Randomized pointer-chasing to defeat prefetching and reuse

If you sequentially access addresses, the GPU’s hardware prefetchers can bring data into cache before you measure it — artificially reducing measured latency.

So, instead of simple A[i] accesses, they used a pointer-chasing pattern:

ld.global.cg.u64 %r1, [%r1];

Here, each load reads the next address to load.
Since the next address is only known after the previous load completes,

- prefetchers cannot predict or issue the next request early,

- and caches can’t help (each load touches a seemingly random, one-time address).

This is the same technique used in CPU and prior GPU latency studies — it serializes dependent loads so that every iteration truly goes out to DRAM.

🧠 5. Step 4 – Using volatile qualifiers and dependent results

To make absolutely sure the compiler or hardware doesn’t reorder or elide memory accesses:

- They marked the loads as volatile.

- ld.global.cv.u64 %r1, [%r1];

Volatile loads cannot be optimized away or replaced with register reuse.

- Each new address depends on the previous one, so there is a strict read-after-write (RAW) dependency chain, ensuring true serialization.

This guarantees that no two loads overlap — each must wait for the last to complete.

⏱️ 6. Step 5 – Timing isolation

They wrapped the pointer-chase loop with clock64() before and after:

unsigned long long start = clock64();

for (int i = 0; i < N; i++) {

asm volatile ("ld.global.cg.u64 %0, [%0];" : "+l"(ptr));

}

unsigned long long end = clock64();

latency = (end - start) / N;

Because each iteration is dependent, (end − start)/N directly gives the per-access latency.

They launched a single thread or warp, so no other warp could hide or overlap the memory latency.
This ensures the measured delay represents the true DRAM round-trip time per access.

🔍 7. Step 6 – Cross-validation with smaller datasets

To verify that caches were indeed bypassed, they repeated the same benchmark with different array sizes:

| Dataset size | Expected hit level | Observed latency |
| --- | --- | --- |
| < L1 size (~128 KB) | L1 hits | ~32 cycles |
| ≈ L2 size (~6 MB) | L2 hits | ~220 cycles |
| ≫ L2 size (hundreds MB) | DRAM | ~480 cycles |

They observed discrete latency plateaus, confirming that as the dataset exceeded each cache level, the latency jumped to the next plateau — proof that the caches were being exhausted and misses were occurring.

🧾 8. Step 7 – Validation via access modifiers and memory policies

They also cross-checked results using alternative PTX cache operators:

- ld.global.ca (cache all)

- ld.global.cg (bypass L1)

- ld.global.cs (streaming)

By comparing the latencies, they confirmed:

- The .ca version matched L2 latency (~220 cycles).

- The .cg version matched DRAM latency (~480 cycles).

This differential validated that their “DRAM latency” case truly bypassed L2 hits.

⚖️ 9. Summary — how they ensured no caching interference

| Potential contamination | Safeguard used |
| --- | --- |
| L1 hits | .cg modifier to skip L1 cache |
| L2 hits | Working set ≫ L2 size |
| Prefetching | Randomized pointer-chasing |
| Out-of-order overlap | RAW dependencies (serialization) |
| Dead-code elimination | volatile + result dependence |
| Compiler optimization | Inline PTX with asm volatile |
| Validation | Stepwise latency vs array size |

✅ The result: each memory load was forced to go all the way to DRAM, producing accurate global-memory latency numbers.

🧠 Takeaway

The key to clean DRAM latency measurements is to remove every possible source of reuse or overlap.
The authors achieved this by combining pointer chasing, cache bypassing qualifiers, huge random working sets, and volatile inline PTX.
Each load depended on the previous, guaranteeing a fully serialized chain of true DRAM accesses — unaffected by any level of caching.

If you’d like, I can show a visual diagram of their pointer-chasing memory-hierarchy test — it’s a simple but elegant depiction of how a single dependent chain forces a complete trip through L1 → L2 → DRAM each time. Would you like that?